/*!

\page tui_transformation_operations_page Transformation Operations

\anchor tui_translation

<br><h2>Translation</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create a vertex and a vector

p1 = geompy.MakeVertex(10, 40, 0)

p2 = geompy.MakeVertex( 0,  0, 50)

p3 = geompy.MakeVertex(50, 80, 0)

v = geompy.MakeVector(p1, p2)

vt = geompy.MakeVector(p1, p3)

# create a cylinder

height = 35

radius1 = 20

cylinder = geompy.MakeCylinder(p1, v, radius1, height)

# translate the given object along the vector, specified by its end points

# (all three functions produce the same result)

translation1 = geompy.MakeTranslationTwoPoints(cylinder, p1, p3)

translation2 = geompy.MakeTranslation(cylinder, 40, 40, 0)

translation3 = geompy.MakeTranslationVector(cylinder, vt)

translation4 = geompy.MakeTranslationVectorDistance(cylinder, vt, 200)

# add objects in the study

id_cylinder = geompy.addToStudy(cylinder, "Cylinder")

id_translation1 = geompy.addToStudy(translation1, "Translation1")

id_translation2 = geompy.addToStudy(translation2, "Translation2")

id_translation3 = geompy.addToStudy(translation3, "Translation3")

id_translation4 = geompy.addToStudy(translation4, "Translation4")

# display the results

gg.createAndDisplayGO(id_cylinder)

gg.setDisplayMode(id_cylinder,1)

gg.createAndDisplayGO(id_translation1)

gg.setDisplayMode(id_translation1,1)

gg.createAndDisplayGO(id_translation2)

gg.setDisplayMode(id_translation2,1)

gg.createAndDisplayGO(id_translation3)

gg.setDisplayMode(id_translation3,1)

gg.createAndDisplayGO(id_translation4)

gg.setDisplayMode(id_translation4,1)

\endcode

\anchor tui_rotation

<br><h2>Rotation</h2>

\code

import geompy

import salome

import math

gg = salome.ImportComponentGUI("GEOM")

# create a vertex and a vector

p1 = geompy.MakeVertex(10, 40,  0)

p2 = geompy.MakeVertex( 0,  0, 50)

p3 = geompy.MakeVertex(10, 50,-20)

p4 = geompy.MakeVertex(10, 50, 60)

v = geompy.MakeVector(p1, p2)

vr = geompy.MakeVector(p3, p4)

# create a cylinder

height = 35

radius1 = 20

cylinder = geompy.MakeCylinder(p1, v, radius1, height)

# rotate the given object around the given axis by the given angle

rotation1 = geompy.MakeRotation(cylinder, vr, math.pi)

rotation2 = geompy.MakeRotationThreePoints(cylinder, p4, p1, p2)

# add objects in the study

id_vr = geompy.addToStudy(vr, "Rotation 1 axis")

id_p4 = geompy.addToStudy(p4, "Rotation 2 center")

id_p1 = geompy.addToStudy(p1, "Rotation 2 point 1")

id_p2 = geompy.addToStudy(p2, "Rotation 2 point 2")

id_cylinder = geompy.addToStudy(cylinder, "Cylinder")

id_rotation1 = geompy.addToStudy(rotation1, "Rotation 1")

id_rotation2 = geompy.addToStudy(rotation2, "Rotation 2")

# display the results

gg.createAndDisplayGO(id_vr)

gg.createAndDisplayGO(id_p4)

gg.createAndDisplayGO(id_p1)

gg.createAndDisplayGO(id_p2)

gg.createAndDisplayGO(id_cylinder)

gg.setDisplayMode(id_cylinder,1)

gg.createAndDisplayGO(id_rotation1)

gg.createAndDisplayGO(id_rotation2)

gg.setDisplayMode(id_rotation1,1)

gg.setDisplayMode(id_rotation2,1)

\endcode

\anchor tui_modify_location

<br><h2>Modify Location</h2>

\code

import geompy

import salome

import math

gg = salome.ImportComponentGUI("GEOM")

# create a vertex and a vector

p1 = geompy.MakeVertex(10, 40, 0)

p2 = geompy.MakeVertex( 0,  0, 50)

v = geompy.MakeVector(p1, p2)

# create a cylinder

height = 35

radius1 = 20

cylinder = geompy.MakeCylinder(p1, v, radius1, height)

circle   = geompy.MakeCircle(p2, v, radius1)

# create local coordinate systems

cs1 = geompy.MakeMarker( 0, 0, 0, 1,0,0, 0,1,0)

cs2 = geompy.MakeMarker(30,40,40, 1,0,0, 0,1,0)

# modify the location of the given object

position = geompy.MakePosition(cylinder, cs1, cs2)

position2 = geompy.PositionAlongPath(position, circle, 0.75, 1, 1)

# add objects in the study

id_cs1 = geompy.addToStudy(cs1, "Coordinate system 1")

id_cs2 = geompy.addToStudy(cs2, "Coordinate system 2")

id_cylinder = geompy.addToStudy(cylinder, "Cylinder")

id_circle = geompy.addToStudy(circle, "Circle")

id_position = geompy.addToStudy(position, "Position")

id_position2 = geompy.addToStudy(position2, "PositionAlongPath")

# display the results

gg.createAndDisplayGO(id_cylinder)

gg.setDisplayMode(id_cylinder,1)

gg.createAndDisplayGO(id_position)

gg.setDisplayMode(id_position,1)

gg.createAndDisplayGO(id_circle)

gg.setDisplayMode(id_circle,1)

gg.createAndDisplayGO(id_position2)

gg.setDisplayMode(id_position2,1)

\endcode

\anchor tui_mirror

<br><h2>Mirror Image</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create a box

box = geompy.MakeBoxDXDYDZ(200, 200, 200)

# create an object, symmetrical to another object through the given plane

p1 = geompy.MakeVertex( 0, 25,  0)

p2 = geompy.MakeVertex( 5, 25,  0)

p3 = geompy.MakeVertex( 0,-30, 40)

plane = geompy.MakePlaneThreePnt(p1, p2, p3, 1000.)

mirror1 = geompy.MakeMirrorByPlane(box, plane)

# create an object, symmetrical to another object through the given axis

p4 = geompy.MakeVertex( 210, 210, -20)

p5 = geompy.MakeVertex( 210, 210, 220)

axis = geompy.MakeVector(p4, p5)

mirror2 = geompy.MakeMirrorByAxis(box, axis)

# create an object, symmetrical to another object through the given point

mirror3 = geompy.MakeMirrorByPoint(box, p4)

# add objects in the study

id_box = geompy.addToStudy(box, "Box")

id_plane = geompy.addToStudy(plane, "Plane")

id_mirror1 = geompy.addToStudy(mirror1, "Mirror plane")

id_axis = geompy.addToStudy(axis, "Axis")

id_mirror2 = geompy.addToStudy(mirror2, "Mirror axis")

id_p4 = geompy.addToStudy(p4, "Point")

id_mirror3 = geompy.addToStudy(mirror3, "Mirror point")

# display the results

gg.createAndDisplayGO(id_box)

gg.setDisplayMode(id_box,1)

gg.createAndDisplayGO(id_plane)

gg.createAndDisplayGO(id_mirror1)

gg.setDisplayMode(id_mirror1,1)

gg.createAndDisplayGO(id_axis)

gg.createAndDisplayGO(id_mirror2)

gg.setDisplayMode(id_mirror2,1)

gg.createAndDisplayGO(id_p4)

gg.createAndDisplayGO(id_mirror3)

gg.setDisplayMode(id_mirror3,1)

\endcode

\anchor tui_scale

<br><h2>Scale Transform</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create a box and a sphere

box = geompy.MakeBoxDXDYDZ(200, 200, 200)

# scale the given object by the factor

p0 = geompy.MakeVertex(100, 100, 100)

factor = 0.5

scale = geompy.MakeScaleTransform(box, p0, factor)

# add objects in the study

id_box = geompy.addToStudy(box, "Box")

id_scale = geompy.addToStudy(scale, "Scale")

# display the results

gg.createAndDisplayGO(id_box)

gg.setDisplayMode(id_box,1)

gg.setTransparency(id_box,0.5)

gg.createAndDisplayGO(id_scale)

gg.setDisplayMode(id_scale,1)

\endcode

\anchor tui_offset

<br><h2>Offset Surface</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create a box and a sphere

box = geompy.MakeBox(20, 20, 20, 200, 200, 200)

# create a new object as offset of the given object

offset = geompy.MakeOffset(box, 70.)

# add objects in the study

id_box = geompy.addToStudy(box, "Box")

id_offset = geompy.addToStudy(offset, "Offset")

# display the results

gg.createAndDisplayGO(id_box)

gg.setDisplayMode(id_box,1)

gg.createAndDisplayGO(id_offset)

\endcode

\anchor tui_multi_translation

<br><h2>Multi Translation</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create vertices and vectors

p0  = geompy.MakeVertex( 0.,  0.,  0.)

px  = geompy.MakeVertex(20.,  0.,  0.)

py  = geompy.MakeVertex( 0., 20.,  0.)

pz  = geompy.MakeVertex( 0.,  0., 20.)

pxy = geompy.MakeVertex( 50., 0., 0.)

pxyz = geompy.MakeVertex( 50., 50., 50.)

vz  = geompy.MakeVector(p0, pz)

vxy = geompy.MakeVector(px, py)

vtr1d = geompy.MakeVector(p0, pxyz)

vtr2d = geompy.MakeVector(p0, pxy)

# create an arc

arc = geompy.MakeArc(py, pz, px)

# create a wire

wire = geompy.MakeWire([vxy, arc])

# create a planar face

face = geompy.MakeFace(wire, 1)

# create a prism

prism = geompy.MakePrismVecH(face, vz, 20.0)

# translate the given object along the given vector a given number of times

tr1d = geompy.MakeMultiTranslation1D(prism, vtr1d, 20, 4)

# consequently apply two specified translations to the object a given number of times

tr2d = geompy.MakeMultiTranslation2D(prism, vtr1d, 20, 4, vtr2d, 80, 3)

# add objects in the study

id_prism = geompy.addToStudy(prism,"Prism")

id_tr1d = geompy.addToStudy(tr1d,"Translation 1D")

id_tr2d = geompy.addToStudy(tr2d,"Translation 2D")

# display the prism and the results of fillet operation

gg.createAndDisplayGO(id_prism)

gg.setDisplayMode(id_prism,1)

gg.createAndDisplayGO(id_tr1d)

gg.setDisplayMode(id_tr1d,1)

gg.createAndDisplayGO(id_tr2d)

gg.setDisplayMode(id_tr2d,1)

\endcode

\anchor tui_multi_rotation

<br><h2>Multi Rotation</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create vertices and vectors

p0  = geompy.MakeVertex( 0.,  0.,  0.)

px  = geompy.MakeVertex(20.,  0.,  0.)

py  = geompy.MakeVertex( 0., 20.,  0.)

pz  = geompy.MakeVertex( 0.,  0., 20.)

pxyz = geompy.MakeVertex( 50., 50., 10.)

vz  = geompy.MakeVector(p0, pz)

vxy = geompy.MakeVector(px, py)

vrot1d = geompy.MakeVector(p0, pxyz)

# create an arc

arc = geompy.MakeArc(py, pz, px)

# create a wire

wire = geompy.MakeWire([vxy, arc])

# create a planar face

face = geompy.MakeFace(wire, 1)

# create a prism

prism = geompy.MakePrismVecH(face, vz, 20.0)

# rotate the given object around the given axis by the given angle a given number of times

rot1d = geompy.MultiRotate1D(prism, vrot1d, 4)

# rotate the given object around the given axis by the given angle a given number of times

# and multi-translate the result of each rotation

rot2d = geompy.MultiRotate2D(prism, vrot1d, 60, 4, 50, 5)

# add objects in the study

id_prism = geompy.addToStudy(prism,"Prism")

id_rot1d = geompy.addToStudy(rot1d,"Rotation 1D")

id_rot2d = geompy.addToStudy(rot2d,"Rotation 2D")

# display the prism and the results of fillet operation

gg.createAndDisplayGO(id_prism)

gg.setDisplayMode(id_prism,1)

gg.createAndDisplayGO(id_rot1d)

gg.setDisplayMode(id_rot1d,1)

gg.createAndDisplayGO(id_rot2d)

gg.setDisplayMode(id_rot2d,1)

\endcode

\anchor tui_fillet2d

<br><h2>Fillet 2D</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create a face in OXY plane

face = geompy.MakeFaceHW(100, 100, 1)

fillet2d = geompy.MakeFillet2D(face, 30, [7, 9])

# add objects in the study

id_face  = geompy.addToStudy(face,"Face_1")

id_fillet2d  = geompy.addToStudy(fillet2d,"Fillet 2D_1")

# display disks

gg.createAndDisplayGO(id_face)

gg.createAndDisplayGO(id_fillet2d)

\endcode

\anchor tui_fillet1d

<br><h2>Fillet 1D</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

# create box

Box_1 = geompy.MakeBoxDXDYDZ(200, 200, 200)

# take box edges to create custom complex wire

[Edge_1,Edge_2,Edge_3,Edge_4,Edge_5,Edge_6,Edge_7,Edge_8,Edge_9,Edge_10,Edge_11,Edge_12] = geompy.SubShapeAllSorted(Box_1, geompy.ShapeType["EDGE"])

# create wire

Wire_1 = geompy.MakeWire([Edge_12, Edge_7, Edge_11, Edge_6, Edge_1,Edge_4])

# make fillet at given wire vertices with giver radius

Fillet_1D_1 = geompy.MakeFillet1D(Wire_1, 55, [3, 4, 6, 8, 10])

# display disks

gg.createAndDisplayGO(Wire_1)

gg.createAndDisplayGO(Fillet_1D_1)

\endcode

\anchor tui_fillet

<br><h2>Fillet</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

radius  = 10.

ShapeTypeEdge = geompy.ShapeType["EDGE"]

# create vertices and vectors

p0  = geompy.MakeVertex(  0.,   0.,   0.)

px  = geompy.MakeVertex(100.,   0.,   0.)

py  = geompy.MakeVertex(  0., 100.,   0.)

pz  = geompy.MakeVertex(  0.,   0., 100.)

vz  = geompy.MakeVector(p0, pz)

vxy = geompy.MakeVector(px, py)

# create an arc

arc = geompy.MakeArc(py, pz, px)

# create a wire

wire = geompy.MakeWire([vxy, arc])

# create a planar face

face = geompy.MakeFace(wire, 1)

# create a prism

prism = geompy.MakePrismVecH(face, vz, 100.0)

# get the list of IDs (IDList) for the fillet

prism_edges = geompy.SubShapeAllSorted(prism, ShapeTypeEdge)

IDlist_e = []

IDlist_e.append(geompy.GetSubShapeID(prism, prism_edges[0]))

IDlist_e.append(geompy.GetSubShapeID(prism, prism_edges[1]))

IDlist_e.append(geompy.GetSubShapeID(prism, prism_edges[2]))

# make a fillet on the specified edges of the given shape

fillet = geompy.MakeFillet(prism, radius, ShapeTypeEdge, IDlist_e)

# make a fillet on all edges of the given shape

filletall = geompy.MakeFilletAll(prism, radius)

# add objects in the study

id_prism = geompy.addToStudy(prism,"Prism")

id_fillet = geompy.addToStudy(fillet,"Fillet")

id_filletall = geompy.addToStudy(filletall,"Fillet all")

# display the prism and the results of fillet operation

gg.createAndDisplayGO(id_prism)

gg.setDisplayMode(id_prism,1)

gg.createAndDisplayGO(id_fillet)

gg.setDisplayMode(id_fillet,1)

gg.createAndDisplayGO(id_filletall)

gg.setDisplayMode(id_filletall,1)

\endcode

\anchor tui_chamfer

<br><h2>Chamfer</h2>

\code

import geompy

import salome

gg = salome.ImportComponentGUI("GEOM")

d1 = 10.

d2 = 10.

ShapeTypeFace = geompy.ShapeType["FACE"]

# create vertices and vectors

p0  = geompy.MakeVertex(  0.,   0.,   0.)

px  = geompy.MakeVertex(100.,   0.,   0.)

py  = geompy.MakeVertex(  0., 100.,   0.)

pz  = geompy.MakeVertex(  0.,   0., 100.)

vz  = geompy.MakeVector(p0, pz)

vxy = geompy.MakeVector(px, py)

# create an arc

arc = geompy.MakeArc(py, pz, px)

# create a wire

wire = geompy.MakeWire([vxy, arc])

# create a planar face

face = geompy.MakeFace(wire, 1)

# create a prism

prism = geompy.MakePrismVecH(face, vz, 100.0)

# get the list of IDs (IDList) for the chamfer

prism_faces = geompy.SubShapeAllSorted(prism, ShapeTypeFace)

f_ind_1 = geompy.GetSubShapeID(prism, prism_faces[0])

f_ind_2 = geompy.GetSubShapeID(prism, prism_faces[1])

IDlist_f = [f_ind_1, f_ind_2]

# perform a chamfer on the edges common to the specified faces

chamfer_e = geompy.MakeChamferEdge(prism, d1, d2, f_ind_1, f_ind_2)

# perform a chamfer on all edges of the specified faces

chamfer_f = geompy.MakeChamferFaces(prism, d1, d2, IDlist_f)

chamfer_f1 = geompy.MakeChamfer(prism, d1, d2, ShapeTypeFace, IDlist_f)

# perform a symmetric chamfer on all edges of the given shape

chamfer_all = geompy.MakeChamferAll(prism, d1)

# add objects in the study

id_prism = geompy.addToStudy(prism,"Prism")

id_chamfer_e = geompy.addToStudy(chamfer_e,"Chamfer edge")

id_chamfer_f = geompy.addToStudy(chamfer_f,"Chamfer faces")

id_chamfer_f1 = geompy.addToStudy(chamfer_f1,"Chamfer faces 1")

id_chamfer_all = geompy.addToStudy(chamfer_all,"Chamfer all")

# display the prism and the results of chamfer operation

gg.createAndDisplayGO(id_prism)

gg.setDisplayMode(id_prism,1)

gg.createAndDisplayGO(id_chamfer_e)

gg.setDisplayMode(id_chamfer_e,1)

gg.createAndDisplayGO(id_chamfer_f)

gg.setDisplayMode(id_chamfer_f,1)

gg.createAndDisplayGO(id_chamfer_f1)

gg.setDisplayMode(id_chamfer_f1,1)

gg.createAndDisplayGO(id_chamfer_all)

gg.setDisplayMode(id_chamfer_all,1)

\endcode

*/